- SSFR-7* – Floor mounted electrically heat traced emergency eyewash and shower safety station with top inlet, ABS plastic shower head with internal 20 GPM flow control, 1” IPS U.S. made stay-open brass ball valve with stainless steel actuating arm, ½” IPS U.S. made 3-way self draining stay-open brass ball valve, 1¼” IPS Schedule 40 hot dipped galvanized pipe and fittings, polished stainless steel pull rod, stainless steel flag handle and powder-coated cast aluminum floor flange, self regulating electric heat-traced cable, UV-resistant ABS plastic jacket with foam insulated core and weather sealed with elastomeric material, 120VAC, 60 HZ single phase power supply, and pipe sensing thermostat (optional freeze protection valve, “FP”).  Trace indicator light included. Unit shall have freeze-resistant spray heads with dust covers.  Spray heads shall be mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant signs.

- Performance: Unit shall be 100% assembled, pressure tested, crated and tested to meet or exceed ANSI Z358.1-2014.  Rated for Class I, Division I, Groups B, C, D. Unit comes with a full 2-year warranty.

- Fixture: Guardian Equipment GFR3200 and GFR3200FP