- SSFR-3* – Floor mounted electrically heat-traced emergency eyewash and shower safety station with top inlet, stainless steel shower head with internal 20 GPM flow control, 1” IPS U.S. made stay-open brass ball valve with stainless steel actuating arm, ½” IPS U.S. made 3-way self draining stay-open brass ball valve, 1¼” IPS Schedule 40 type 304 stainless steel pipe and fittings, polished stainless steel pull rod, stainless steel flag handle and powder-coated cast aluminum floor flange, self regulating electric heat-traced cable, UV-resistant ABS plastic jacket with foam insulated core, 120VAC, 60 HZ single phase power supply, and pipe sensing thermostat (optional freeze protection valve, “FP”).  Unit shall have freeze-resistant spray heads with dust covers.  Spray heads shall be mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant signs.

- Performance: Unit shall be 100% assembled, pressure tested, crated and tested to meet or exceed ANSI Z358.1-2014.  Unit comes with a full 2-year warranty.

- Fixture: Guardian Equipment GFR3500 and GFR3500FP